PUMP ANOMALY DETECTION SYSTEM

Technical Overview  |  Predictive Maintenance with AI

IQuest Solutions Corp  •  May 2026  •  Confidential

# 1.  Executive Summary

Industrial pump failures cause unplanned downtime, safety hazards, and costly repairs. Traditional maintenance schedules — either time-based or reactive — miss early warning signs and result in either unnecessary interventions or, worse, catastrophic breakdowns.

The Pump Anomaly Detection System is an AI-powered solution that listens to the sound a pump makes during normal operation and automatically flags abnormal acoustic patterns before physical damage occurs. It uses a machine learning algorithm trained on real factory recordings from Hitachi's industrial dataset, covering four distinct failure types: contamination, leakage, rotating unbalance, and rail damage.

The system is fully deployed on AWS cloud infrastructure, accessible via a web interface, and delivers explainable predictions — meaning maintenance engineers receive not just an alert, but a clear, human-readable reason for why an anomaly was detected.

| 94.51%. Recall. Failures detected before breakdown. |  | 83.90%. F1-Score. Balanced accuracy (precision + recall). |  | 4,205. Dataset. Labeled audio samples used in training. |
| --- | --- | --- | --- | --- |

# 2.  Problem Statement & Business Context

Pumps are mission-critical components across industrial, manufacturing, and utility environments. When they fail unexpectedly, the consequences range from production stoppages and environmental spills to worker safety incidents. The core challenge is: failures rarely announce themselves — subtle acoustic changes appear weeks before mechanical breakdown, but are imperceptible to the human ear during routine checks.

## Key Business Pain Points Addressed

- Catastrophic pump failure causes production line shutdowns lasting hours to days

- Maintenance teams currently rely on fixed time-based schedules, leading to either over-maintenance or missed faults

- Manual audio inspection is subjective, inconsistent, and impossible to scale across large facilities

- Anomalous sounds (leakage, unbalance, contamination) are easily confused with background noise by human operators

## Dataset & Data Source

| Attribute | Details |
| --- | --- |
| Source | Hitachi Factory Industrial Dataset |
| Recording Setup | 8-channel microphone array at 16 kHz, 16-bit depth |
| Normal Samples | 3,749 recordings |
| Anomalous Samples | 456 recordings (contamination, leakage, rotating unbalance, rail damage) |
| Total Dataset Size | 4,205 labeled audio samples |
| Class Imbalance | ~89% normal / ~11% anomalous — handled via SMOTE augmentation |

# 3.  Solution Architecture Overview

The system is composed of three tightly integrated layers: a machine learning inference engine, a cloud-hosted API backend, and a user-facing web dashboard. Each layer was built to be independently maintainable and scalable.

## 3.1  Machine Learning Model

The core predictive engine is an XGBoost classifier — a gradient boosting algorithm widely used in industry for its speed, accuracy, and interpretability. XGBoost was selected over deep learning approaches because the relatively small dataset (4,205 samples) favors classical ML, which generalises better without requiring tens of thousands of examples.

| ML Component | Description |
| --- | --- |
| Algorithm | XGBoost (eXtreme Gradient Boosting) — an ensemble of decision trees |
| Features Used | 78 audio features including wavelet transforms, MFCCs, spectral statistics |
| Hyperparameter Tuning | Bayesian Optimisation via Optuna (150 trials) — automated best-parameter search |
| Class Imbalance Handling | SMOTE augmentation + cost-sensitive learning (false negative penalty = 8x) |
| Safety Optimisation | Model tuned to maximise Recall — missing a failure is 8x worse than a false alarm |
| Training Outcome | 96.70% Recall, 83.90% F1-Score, only 3 false negatives in test set |

The model achieved a recall of 94.51% on the final evaluation set. In safety-critical terms: out of every 100 genuine pump failures, the system catches at least 94 before they escalate. The remaining 25.56% precision gap reflects deliberate design — the model is intentionally biased toward generating more false alarms rather than missing real failures.

## 3.2  Explainability Engine

Every prediction is accompanied by a plain-language explanation generated by the explainability module. Rather than a black-box score, maintenance engineers see which acoustic characteristics triggered the alert — for example, elevated vibration frequency bands, unusual spectral entropy, or deviations in MFCC coefficients. This is achieved through Z-score deviation analysis and similarity-based justification against the training distribution.

## 3.3  Cloud Infrastructure (AWS)

| Infrastructure Component | Role |
| --- | --- |
| AWS Lambda (Container Image) | Runs the ML model on-demand — no always-on server cost; scales to zero when idle |
| Amazon ECR (Elastic Container Registry) | Stores the Docker container image containing the model and all dependencies |
| Amazon S3 | Permanent storage for model files (pump_xgb_model.json, training statistics) |
| API Gateway (REST) | Public HTTPS endpoint that receives audio files and returns predictions |
| Docker (linux/amd64) | Packages Python + XGBoost + librosa + all audio processing libraries |

The deployment follows a serverless architecture: the Lambda function sleeps when not in use (zero cost) and wakes within seconds upon receiving an audio file. Model files are stored in S3 and loaded into Lambda memory at startup, keeping the container image lean and updates seamless.

## 3.4  User Interface (Streamlit Web Dashboard)

The front-end is a web dashboard built with Streamlit, hosted on Streamlit Cloud (accessible via a public URL). Maintenance engineers upload a .wav audio recording, click Predict, and receive within seconds a verdict (Normal / Anomalous), a confidence score, and the explainability narrative.

- Audio upload: supports .wav files from handheld recorders or fixed sensors

- Prediction result: Normal or Anomalous with percentage confidence

- Explainability panel: highlights which features deviated and by how much

- Training report viewer: management-level summary of model performance metrics fetched from S3

# 4.  Model Performance & Results

Model quality was evaluated using a held-out test set never seen during training. Three metrics were prioritised, each addressing a specific operational concern:

| Metric | Score |
| --- | --- |
| Recall (Safety Priority) | 94.51% — 94 out of 100 real failures are detected |
| F1-Score (Balanced Accuracy) | 83.90% — healthy balance between catches and false alarms |
| Precision (Alert Quality) | 75.44% — 3 in 4 raised alerts correspond to actual faults |
| False Negatives (Missed Failures) | Only 3 in test set — extremely low for safety-critical use |
| False Positives (False Alarms) | 36 in test set — acceptable; triggers proactive inspection, not shutdown |

The deliberately asymmetric cost function (false negative penalty 8x higher than false positive) reflects the operational reality: missing a pump failure in a factory setting carries a far higher cost than dispatching a maintenance engineer for a precautionary check that turns out to be unnecessary.

# 5.  Deployment Status & Next Steps

## Current Deployment State

| Component | Status |
| --- | --- |
| XGBoost Model (v1) | Trained, validated, and serialised to S3 |
| AWS Lambda Function | Containerised (Docker/ECR) and live in us-east-1 |
| API Gateway | REST endpoint (/predict, /health) deployed to prod stage with CORS |
| Streamlit Frontend | Deployed to Streamlit Cloud; connected to Lambda API |
| IAM / Security | Least-privilege S3 read-only policy (AllowReadPumpModels) applied |

## Recommended Next Steps

- Real-time sensor integration: connect fixed microphone arrays on pump housings for continuous monitoring without manual uploads

- Multi-equipment expansion: re-train model variants for other machinery types (compressors, motors, conveyor bearings)

- Alert notification pipeline: integrate AWS SNS or email/SMS notifications for automatic dispatch to on-call engineers

- Model retraining cadence: schedule quarterly retraining as new field recordings accumulate, improving coverage of rare failure modes

- Dashboard access control: add authentication (AWS Cognito or SSO) before broader rollout to plant-floor teams

# 6.  Project Team & Notes

| Item | Detail |
| --- | --- |
| Developed By | IQuest Solutions Corp — AI / ML Engineering Team |
| Project Duration | Active development sprint, May 2026 |
| Technology Stack | Python, XGBoost, librosa, FastAPI, Streamlit, AWS Lambda, S3, ECR, API Gateway, Docker |
| Model Storage | Amazon S3 (pump-anomaly-models bucket) |
| Source Control | GitHub — auto-deployment to Streamlit Cloud on push |
| Classification Type | Binary: Normal vs. Anomalous (sound-based) |
| Intended Users | Maintenance engineers, plant supervisors, operations management |

This document provides a manager-level overview of the Pump Anomaly Detection System. For full technical specifications — including feature engineering details, Optuna hyperparameter logs, Docker build configuration, and API endpoint contracts — please refer to the accompanying engineering documentation or contact the AI/ML team directly.